AUTOMOTIVE TECHNICAL DOCUMENTATION
Robotic Resistance Spot Welding
for Body-in-White (BIW) Manufacturing

Research Report and Technical Documentation Roadmap

| Item | Details |
| --- | --- |
| Prepared by | Kuganraj K |
| Topic | Robotic Resistance Spot Welding (RSW) in Automotive Body-in-White |
| Document type | Automotive Technical Research Report + Planning Strategy |
| Research scope | Technology, process, robot integration, quality, safety, standards, trends and future roadmap |
| Planned effort | 30–35 hours |
| Research cut-off | 8 October 2026 |

Prepared for technical documentation coursework
Publicly available sources were reviewed; proprietary OEM information is not used.

# Document Control and Submission Checklist

| Requirement | How this report addresses it | Status |
| --- | --- | --- |
| Specific automotive technology | Robotic resistance spot welding for BIW assembly | Complete |
| Research report | Technology fundamentals, process, equipment, quality, safety and applications | Complete |
| Literature review | Peer-reviewed research plus standards bodies and industry sources | Complete |
| Current trends | AI/ML quality prediction, digital twins, online monitoring, flexible multi-robot cells and aluminum RSW | Complete |
| Technical specifications | Key process variables, robot/cell elements, quality indicators and measurement considerations | Complete |
| Industry standards | ISO 10218, ISO 14373, ISO 17657, ISO 9283 and AWS D8 documents | Complete |
| Structured roadmap | Final document architecture, writing sequence, review gates and deliverables | Complete |
| Timeline | 30–35 hour plan with milestones | Complete |
| Citations | Numbered in-text citations and source list with links | Complete |
| Professional formatting | Cover, headings, tables, references, header/footer and consistent typography | Complete |

# Table of Contents / Report Map

1. Executive Summary

2. Research Objective and Scope

3. Technology Background: Robotic Resistance Spot Welding

4. Technical Process and System Architecture

5. Technical Specifications and Key Process Variables

6. Literature Review and Research Findings

7. Current Industry Trends

8. Quality Assurance, Inspection and Data Analytics

9. Safety and Industry Standards

10. Benefits, Limitations and Implementation Considerations

11. Proposed Technical Document Structure

12. Documentation Roadmap and Writing Strategy

13. 30–35 Hour Timeline and Milestones

14. Conclusion

15. References

# 1. Executive Summary

Robotic resistance spot welding (RSW) is a foundational joining technology in automotive Body-in-White (BIW) manufacturing. The process combines electrical resistance heating, electrode force, controlled weld time, fixtures and industrial robots to create repeatable spot welds at production scale. Research shows that RSW remains highly relevant because it is fast, automatable and well suited to high-volume sheet-metal assembly, while current development is moving toward flexible multi-robot cells, digital process planning, online sensing, machine learning and digital twins. [1][2][3][4]

This report therefore proposes a technical-documentation project focused on “Robotic Resistance Spot Welding for Automotive BIW: Process, Robot Programming, Quality, Safety and Intelligent Manufacturing.” The final technical document should explain the process from fundamentals through production implementation, while separating established engineering practice from emerging research.

The standards review identifies ISO 14373:2024 as a current international procedure for resistance spot welding of specified low-carbon steels, ISO 17657-1:2005 as a current reference for welding-current measurement, and ISO 10218-1:2025 / ISO 10218-2:2025 as the current robot safety framework for industrial robots and robot applications/cells. AWS D8.1M:2021 provides automotive weld-quality criteria for resistance spot welding of steel, AWS D8.2M:2026 addresses aluminum, and AWS D8.9M:2022 provides test methods for automotive sheet-steel weldability. [5][6][7][8][9][10]

A 30–35 hour documentation plan is proposed. The work is divided into research, technical analysis, outline development, drafting, standards verification, editing, citation checking and final quality assurance. The roadmap is designed to produce a technically credible document suitable for engineering learning, manufacturing documentation and future expansion into a detailed standard operating or process-engineering guide.

# 2. Research Objective and Scope

## 2.1 Objective

The objective is to research robotic resistance spot welding in automotive BIW manufacturing and create a defensible plan for a comprehensive technical document. The work must explain what the technology is, why it is used, how a robotic welding cell operates, how weld quality is controlled, which standards govern relevant aspects, and how emerging digital technologies are changing the process.

## 2.2 Research Questions

How does resistance spot welding create a joint between automotive sheet-metal components?

What roles do current, electrode force, weld time, material, coating, electrode condition and cooling play in weld quality?

How are industrial robots, servo welding guns, fixtures and controllers integrated into a BIW welding cell?

Which quality indicators and inspection methods are used to verify spot weld performance?

Which ISO and AWS standards are relevant to welding procedures, quality, current measurement, robot safety and robot performance?

What are the most important current trends in digitalization, AI/ML, online monitoring and digital twins?

How should the final technical document be structured so that a technician, engineer or learner can follow it logically?

## 2.3 Scope and Boundaries

| Included | Outside primary scope |
| --- | --- |
| Automotive BIW resistance spot welding | Complete vehicle welding beyond BIW |
| Industrial robot integration and cell architecture | Consumer/service robotics |
| Steel and emerging aluminum RSW considerations | Detailed metallurgy of every automotive alloy |
| Process variables, quality, inspection and maintenance | Proprietary OEM welding schedules |
| Safety and relevant public standards | Legal certification advice for a specific plant |
| Digitalization, AI/ML and digital twins | Production deployment of an unvalidated AI model |

# 3. Technology Background: Robotic Resistance Spot Welding

## 3.1 What is RSW?

Resistance spot welding joins overlapping sheets by passing electrical current through the contact region while electrodes apply mechanical force. The electrical resistance at the joint generates heat; under controlled force and time, a weld nugget forms between the sheets. After current stops, the electrodes remain in contact for a hold period to allow the joint to solidify under pressure. The process is especially attractive for BIW because thousands of repeatable joints can be produced without adding filler metal.

## 3.2 Why Automotive BIW Uses RSW

High production rate: short weld cycles support high-volume assembly.

Automation suitability: robots can repeatedly position welding guns at programmed weld locations.

Process repeatability: controlled current, force and time can produce consistent joints when equipment and materials are controlled.

Packaging flexibility: spot welds can join overlapping sheet components without requiring access to the full perimeter.

Compatibility with fixtures and line automation: robots, clamps, conveyors and body fixtures can be coordinated as one production system.

Published research describes RSW as a dominant joining process in BIW and notes that automotive bodies can contain thousands of spot welds. The exact number varies by vehicle architecture, material mix and OEM design, so a fixed weld-count figure should not be treated as universal. [2][11]

## 3.3 Typical Robotic Cell Elements

| Subsystem | Function | Documentation focus |
| --- | --- | --- |
| Industrial robot | Moves welding gun/tool along programmed paths | Axes, payload, reach, accuracy, frames, TCP |
| Welding gun | Applies electrode force and conducts current | Gun type, stroke, force, electrode geometry |
| Welding controller | Controls current, time and weld sequence | Schedule, feedback, alarms, interlocks |
| Transformer / power unit | Supplies high-current welding power | Electrical rating, cooling, monitoring |
| Electrodes | Transfer force/current to sheets | Material, tip geometry, dressing, wear |
| Fixture | Locates and clamps body components | Datums, clamps, access, repeatability |
| Safety system | Controls safe access and machine states | Fencing, interlocks, E-stops, safety functions |
| PLC / line control | Coordinates station sequence and permissions | I/O, interlocks, handshakes |
| Quality/data layer | Stores process and quality information | Traceability, signals, alarms, analytics |

# 4. Technical Process and System Architecture

## 4.1 Basic Weld Sequence

| Step | Technical description |
| --- | --- |
| Approach | Robot positions the welding gun at the programmed weld point. |
| Squeeze | Electrodes close and establish the required mechanical force. |
| Weld | Current flows for a controlled time; resistance heating creates the nugget. |
| Hold | Current stops while electrode force is maintained to support solidification. |
| Open | Electrodes release the joint. |
| Move | Robot transitions to the next programmed point while the cell maintains sequencing and safety conditions. |

## 4.2 Process-Parameter Relationship

Weld quality is not controlled by one parameter. Electrode force, welding current and weld time interact with sheet thickness, material grade, surface coating, contact resistance, electrode condition and cooling. A simplified engineering concept is that heat generation is related to electrical resistance and current over time; however, production schedules should be established and validated using appropriate material and equipment trials rather than calculated from a single simplified equation.

| Parameter | If too low | If too high / poorly controlled | Typical control approach |
| --- | --- | --- | --- |
| Welding current | Insufficient heating; undersized nugget | Expulsion, electrode wear, surface damage | Controller schedule + current monitoring |
| Weld time | Insufficient heat input | Excessive heat, cycle-time penalty | Programmed weld time |
| Electrode force | Poor contact, instability, expulsion risk | Reduced resistance / undersized nugget in some conditions; mechanical effects | Servo/pneumatic force control |
| Electrode condition | Variable contact and current density | Excessive wear changes tip geometry | Tip dressing and life monitoring |
| Material/coating | Changed contact/resistance behavior | Coating interaction and electrode degradation | Validated weld window |
| Cooling | Overheating and accelerated wear | Overcooling can affect process conditions | Water/thermal monitoring |

## 4.3 Robot Programming and Coordinate Systems

A robotic welding program must define safe motion, approach and retract positions, tool-center-point (TCP) behavior, work/object frames, weld-point locations, gun orientation, sequencing and interlocks. For a technical document, robot programming should be described generically so that the principles apply across manufacturers such as FANUC, ABB, Yaskawa or KUKA without incorrectly presenting one controller's syntax as an industry standard.

Define the robot base/world coordinate system and relevant work/object frames.

Validate the welding-gun TCP and its orientation.

Use approach and retreat positions that respect fixtures, adjacent robots and body geometry.

Sequence weld points to manage access, cycle time and collision risk.

Use appropriate line/PLC handshakes and safety conditions before welding.

Verify offline simulation results against the physical cell before production release.

Maintain version control and change history for robot programs and process schedules.

## 4.4 Multi-Robot Cell Planning

Research on multi-robot spot-welding cells emphasizes that cell design and robot motion planning are coupled engineering problems. The design must consider the fixture, transport device, robot locations, welding-gun selection, weld-point access and collision avoidance. Flexible production has increased the importance of designing cells that can accommodate different vehicle models and production variants. [3]

# 5. Technical Specifications and Key Process Variables

## 5.1 Specification Categories

| Category | Examples of information to capture in the final technical document |
| --- | --- |
| Robot | Manufacturer/model, axes, payload, reach, repeatability, controller, safety functions |
| Welding gun | Servo/pneumatic type, stroke, force range, electrode geometry, cooling |
| Power supply | AC/DC technology, current capability, weld schedule controls, feedback |
| Material | Grade, thickness, coating, stack-up, surface condition |
| Weld schedule | Current, weld time, squeeze/hold timing, force, pulse profile where applicable |
| Electrode management | Tip material, tip geometry, dressing interval, wear limits, cooling |
| Fixture | Datum strategy, clamps, access, body location repeatability |
| Quality | Nugget/weld size, mechanical tests, visual criteria, defect classification, traceability |
| Data | Robot position, current, force, time, alarms, tip life, weld result, vehicle/body ID |

## 5.2 Engineering Caution on Numeric Settings

A professional technical document should distinguish between general process principles and production-specific numeric settings. Welding current, force and time should not be presented as universal values because they depend on material, thickness, coating, stack-up, electrode geometry, equipment and validated process windows. The final document should therefore use representative examples only and instruct readers to use approved welding schedules, validated trials and plant-specific process documentation.

# 6. Literature Review and Research Findings

## 6.1 Research Selection Method

The literature review used a source hierarchy: (1) international standards bodies and professional organizations for requirements and terminology; (2) peer-reviewed journal and conference literature for technical findings; (3) recognized industry publications for current practice and trends; and (4) supplier case studies only where they illustrate implementation rather than establish universal technical requirements.

## 6.2 Key Literature Findings

| Research area | Finding | Source |
| --- | --- | --- |
| Multi-robot cell design | Pellegrinelli et al. show that BIW multi-robot spot-welding requires coordinated cell design and motion planning, with flexibility for different vehicle models as a major production driver. | [3] |
| Digital process planning | Research on BIW spot-welding process planning identifies physical trials as costly and time-consuming and explores virtual testing to reduce development loops; information sharing and workflow continuity are critical. | [4] |
| Quality assurance | A 2024 review reports that many established RSW quality programs rely on offline or destructive practices, while online quality assessment is increasingly feasible and important for higher quality assurance. | [12] |
| Machine learning | A 2024 study evaluated several ML approaches for automotive RSW quality prediction; XGBoost achieved 97.1% accuracy on the reported unseen dataset. This is a research result, not a universal production guarantee. | [13] |
| Digital twins | A 2025 Scientific Reports study used digital-twin-generated dynamic-resistance data with machine learning for automotive RSW quality prediction and reported strong test performance in its case study. | [14] |
| AI vision | Recent research demonstrates computer-vision approaches for non-destructive weld-quality classification using visible and thermal information. | [15] |
| Aluminum RSW | Recent research highlights the increasing use of RSW for aluminum and the sensitivity of weld-spot diameter to welding current, electrode force, adhesive and sheet thickness. | [16] |
| AI in automotive welding | AWS Welding Journal describes advanced sensing and data analytics as important components for improving weld monitoring and quality assurance, including electrode-force and displacement sensing. | [17] |

## 6.3 Literature Review Synthesis

The literature indicates a clear evolution. Traditional RSW documentation concentrates on process windows, equipment settings, electrode management and offline quality checks. Modern research increasingly treats the welding cell as a data-generating cyber-physical system. Sensors, robot/controller signals, dynamic resistance, force/displacement and vision can be combined with analytics to predict quality or identify abnormal conditions. Digital twins can provide virtual data and test scenarios, potentially reducing the dependence on costly physical experiments. The major technical challenge is moving from promising laboratory or case-study models to robust, explainable and maintainable production systems.

# 7. Current Industry Trends

| Trend | Technical significance |
| --- | --- |
| Flexible multi-robot cells | Cell layouts increasingly need to accommodate product variants, changing weld-point access requirements and optimized robot coordination. [3] |
| Offline programming and digital twins | Virtual process planning can reduce physical teaching and commissioning time. Research also identifies digital twins as a path toward virtual process validation and data generation. [4][14] |
| Online weld-quality monitoring | The industry is moving from sample-based inspection toward greater use of process signals and online assessment. [12][17] |
| Machine learning for quality prediction | ML is being investigated for weld-quality classification, defect detection and expulsion prediction. Results can be strong in controlled datasets, but generalization must be validated before production deployment. [13][18] |
| Computer vision | Vision systems are being researched for automated defect detection and weld-feature measurement, potentially supporting non-destructive inspection and maintenance decisions. [15] |
| Aluminum and mixed-material challenges | Aluminum RSW is receiving increased attention because electrical/thermal behavior, coatings, adhesives and electrode interactions differ from traditional steel applications. [16] |
| Closed-loop manufacturing | The long-term direction is to sense process behavior, classify quality, recommend or execute corrective actions, and feed the results back into maintenance and process planning. |

## 7.1 Trend-to-Roadmap Matrix

| Trend | Near-term documentation content | Future extension |
| --- | --- | --- |
| Digital twin | Explain virtual cell, robot paths and process validation | Simulation-linked production data |
| AI/ML | Explain data pipeline and model-validation principles | Real-time prediction and closed-loop control |
| Vision | Describe inspection concept and limitations | 100% non-destructive inspection |
| Online sensing | Document current/force/displacement signals | Adaptive process control |
| Flexible robotics | Document multi-model cell planning | Dynamic scheduling and optimization |

# 8. Quality Assurance, Inspection and Data Analytics

## 8.1 Quality Chain

Material verification → confirm grade, thickness and surface/coating condition.

Equipment verification → confirm robot, gun, controller, transformer and cooling status.

Electrode verification → inspect tip geometry, dressing condition and cooling.

Process verification → validate current, force, time and schedule.

Weld verification → apply appropriate visual, dimensional, mechanical or non-destructive checks.

Traceability → associate process results with body/VIN/serial number and weld location where the production system supports it.

Reaction plan → define what happens when a weld is outside the accepted process/quality condition.

## 8.2 Typical Weld Defects to Cover

| Defect / condition | Possible contributors | Documentation response |
| --- | --- | --- |
| Undersized nugget | Insufficient heat input, poor contact, unsuitable schedule | Check validated weld window and equipment condition |
| Expulsion | Excessive heat/current, force or surface/contact effects | Use process monitoring and validated corrective action |
| Electrode sticking / excessive wear | High heat, coating interaction, inadequate cooling/dressing | Review tip life, dressing and cooling |
| Surface indentation / burn | Electrode geometry or excessive local heat/force | Inspect electrodes and schedule |
| Missing weld | Robot program, controller, interlock or equipment fault | Use traceability and station-level fault logic |
| Mislocated weld | TCP/frame error, body location or program error | Verify coordinates, fixture and robot calibration |

## 8.3 Data Architecture for Intelligent Welding

A future-ready technical document should describe the data path as: physical weld → sensors/controller → edge or PLC layer → manufacturing execution/quality system → analytics/model → decision → maintenance/process response. Potential features include welding current, voltage, dynamic resistance, electrode force, displacement, weld time, robot ID, weld-point ID, tip-life state, material stack-up and inspection result. The data model should preserve traceability and timestamps so that process conditions can be related to quality outcomes.

# 9. Safety and Industry Standards

## 9.1 Standards Landscape

| Standard | Title | Relevance | Ref. |
| --- | --- | --- | --- |
| ISO 10218-1:2025 | Robotics — Safety requirements — Part 1: Industrial robots | Robot-level safety requirements; current 2025 edition. | [5] |
| ISO 10218-2:2025 | Robotics — Safety requirements — Part 2: Industrial robot applications and robot cells | Application/cell integration safety; use alongside Part 1. | [5][19] |
| ISO 14373:2024 | Resistance welding — Procedure for spot welding of uncoated and coated low-carbon steels | Procedure reference covering specified sheet configurations and welding equipment. | [6] |
| ISO 17657-1:2005 | Resistance welding — Welding current measurement — Part 1 | Guidance for measurement/calibration of welding-current measurement systems; confirmed current in 2024. | [7] |
| ISO 9283:1998 | Manipulating industrial robots — Performance criteria and related test methods | Robot performance criteria and testing; current version confirmed in 2021. | [20] |
| AWS D8.1M:2021 | Specification for Automotive Weld Quality — Resistance Spot Welding of Steel | Visual and measurable acceptance criteria for automotive steel spot welds. | [8] |
| AWS D8.2M:2026 | Specification for Automotive Weld Quality — Resistance Spot Welding of Aluminum | Current automotive aluminum spot-weld quality specification. | [9] |
| AWS D8.9M:2022 | Test Methods for Evaluating the Resistance Spot Welding Behavior of Automotive Sheet Steel Materials | Tests for current range, electrode endurance and weld properties. | [10] |

## 9.2 Safety Documentation Topics

Risk assessment for robot motion, welding current, hot surfaces, pinch/crush points, sparks and stored energy.

Safeguarding: physical barriers, gates/interlocks, emergency stops and appropriate safety-rated control functions.

Safe operating modes for teaching, setup, maintenance and automatic production.

Lockout/tagout or the applicable plant energy-isolation procedure during maintenance.

Electrical, pneumatic/hydraulic and thermal energy controls.

Welding fumes, arc flash/brightness, heat and fire risk appropriate to the equipment and process.

Clear distinction between robot manufacturer instructions, plant procedures and international standards.

ISO 10218-1:2025 explicitly addresses industrial robots as partly completed machinery, while ISO 10218-2:2025 covers industrial robot applications and robot cells. The ISO source also notes that application-specific hazards, such as welding hazards, must be addressed during application design. [5][19]

# 10. Benefits, Limitations and Implementation Considerations

| Dimension | Benefits | Limitations / risks |
| --- | --- | --- |
| Productivity | Fast repeatable joining and automation | Downtime can be costly when a cell fault stops production |
| Quality | Controlled schedules and repeatable robot positioning | Quality depends on material, electrodes, equipment and process discipline |
| Flexibility | Programmable robots can support multiple body variants | Programming and commissioning complexity increases |
| Safety | Automation can isolate workers from repetitive welding operations | Robot and welding hazards require engineered safeguards |
| Data | Process signals can support traceability and analytics | Poor data quality can create misleading models |
| AI | Potential for early anomaly/quality detection | Models may not generalize across materials, tools or plants |
| Digital twin | Virtual testing and process planning | Model fidelity and data integration are difficult |

# 11. Proposed Technical Document Structure

The final technical document should be written as a practical engineering guide rather than as a literature-only paper. Each major section should begin with its purpose, explain the engineering concept, provide a table/diagram where useful, identify safety or quality implications, and end with a short takeaway.

| Proposed section | Content |
| --- | --- |
| Front matter | Title page, document control, revision history, contents, list of figures/tables, abbreviations. |
| 1. Introduction | Purpose, audience, scope, assumptions, terminology and document-use instructions. |
| 2. BIW Manufacturing Context | Vehicle body structure, joining strategy and why RSW is used. |
| 3. RSW Fundamentals | Electrical resistance heating, force, current, time, nugget formation and cooling. |
| 4. Robotic Welding Cell | Robot, gun, transformer, controller, fixtures, PLC, safety system and utilities. |
| 5. Robot Programming | TCP, frames, approach/retract, weld-point sequencing, collision avoidance and program control. |
| 6. Weld Schedule and Process Control | Parameter windows, material stack-up, electrode management and process validation. |
| 7. Quality Assurance | Acceptance criteria, inspection, traceability, defect modes and reaction plans. |
| 8. Maintenance | Electrode dressing, tip life, cooling, calibration, preventive/predictive maintenance. |
| 9. Safety | Risk assessment, safeguarding, operating modes, energy isolation and welding hazards. |
| 10. Digitalization and AI | Sensors, MES/edge architecture, analytics, ML, computer vision and digital twins. |
| 11. Standards and Compliance | Applicable ISO/AWS references and how to use them without replacing plant requirements. |
| 12. Troubleshooting | Symptom → probable cause → verification → corrective action. |
| 13. Case Study / Example | Generic BIW workcell example with a safe, non-proprietary process narrative. |
| 14. Conclusion | Key lessons, implementation priorities and future opportunities. |
| Appendices | Glossary, parameter worksheet, inspection checklist, revision history and references. |

# 12. Documentation Roadmap and Writing Strategy

## 12.1 Phase-Gate Approach

| Gate | Main activity | Output |
| --- | --- | --- |
| Gate 1 — Define | Confirm topic, audience, scope and research questions. | Approved scope statement |
| Gate 2 — Research | Collect and classify authoritative standards and literature. | Source matrix + notes |
| Gate 3 — Architect | Build document outline, information hierarchy and figure/table plan. | Approved outline |
| Gate 4 — Draft | Write core technical sections using citations. | First complete draft |
| Gate 5 — Validate | Check technical claims, standards editions, terminology and numerical statements. | Validated draft |
| Gate 6 — Edit | Improve clarity, consistency, diagrams/tables, references and formatting. | Publication-ready draft |
| Gate 7 — QA | Final checklist, file test, citation check and submission packaging. | Final DOCX |

## 12.2 Source Management Strategy

Use standards-body pages as the primary authority for current standard titles, editions and scope.

Use peer-reviewed literature for scientific findings, performance results and emerging methods.

Record the publication year and DOI/URL for every research source.

Do not convert one laboratory result into a universal manufacturing specification.

Mark withdrawn/superseded standards clearly; use current editions in the main compliance discussion.

Keep OEM-specific process schedules and passwords out of a public technical document.

Maintain a claim-to-source matrix so every important technical statement can be traced to evidence.

## 12.3 Planned Figures and Tables

| No. | Planned visual | Purpose |
| --- | --- | --- |
| Figure 1 | Simplified RSW principle diagram | Explain current path, electrode force and nugget formation |
| Figure 2 | Generic robotic BIW cell architecture | Show robot/gun/fixture/PLC/safety relationships |
| Figure 3 | Weld-cycle timing diagram | Explain squeeze–weld–hold–open sequence |
| Figure 4 | Robot programming coordinate concept | Explain TCP and work/object frames |
| Figure 5 | Quality data pipeline | Show sensor → edge/MES → analytics → action |
| Figure 6 | Digital-twin concept | Show physical and virtual process relationship |
| Table 1 | Key process variables | Quick technical reference |
| Table 2 | Defects and corrective actions | Troubleshooting |
| Table 3 | Standards matrix | Compliance/reference overview |

# 13. 30–35 Hour Timeline and Milestones

The task is estimated at approximately 33 hours, within the requested 30–35 hour range. The schedule below is designed for focused individual work and includes review time rather than assuming that drafting is the only activity.

| Stage | Activity | Hours | Work package | Milestone |
| --- | --- | --- | --- | --- |
| 1 | Topic confirmation + scope | 2.0 | Finalize technology topic, audience, research questions and boundaries | Scope approved |
| 2 | Literature search | 4.0 | Find peer-reviewed papers, standards and credible industry sources | Source library |
| 3 | Literature review | 4.0 | Read, compare, classify and summarize findings | Literature matrix |
| 4 | Technical analysis | 4.0 | Map RSW process, robot cell, variables, quality and maintenance | Technical notes |
| 5 | Standards analysis | 3.0 | Verify current ISO/AWS editions and relevance | Standards matrix |
| 6 | Document architecture | 2.0 | Create final outline, figure/table plan and information hierarchy | Approved outline |
| 7 | Drafting | 6.0 | Write introduction, technical sections, trends and standards | Complete first draft |
| 8 | Roadmap + conclusion | 2.0 | Write documentation plan, milestones and conclusion | Planning section |
| 9 | Technical review | 2.0 | Check claims, units, terminology, standards and source alignment | Validated draft |
| 10 | Formatting + final QA | 4.0 | Format Word document, references, headers, tables and submission checklist | Final DOCX |
|  | TOTAL | 33.0 |  | Submission-ready document |

## 13.1 Milestone Acceptance Criteria

M1: Topic and scope are specific enough to prevent the document becoming a general automotive overview.

M2: At least one authoritative standards source supports each standards claim.

M3: Research findings distinguish peer-reviewed evidence from supplier marketing or illustrative case studies.

M4: Every major technical section has a clear purpose and intended audience.

M5: Current trends are linked to engineering implications rather than listed as buzzwords.

M6: Draft contains traceable citations and a complete reference list.

M7: Final Word file passes formatting, spelling, citation, table and submission checks.

# 14. Conclusion

Robotic resistance spot welding is an appropriate and technically rich topic for a comprehensive automotive technical document because it sits at the intersection of manufacturing engineering, welding science, industrial robotics, quality assurance and digital manufacturing. The research reviewed for this plan shows that the technology remains fundamental to BIW assembly while its supporting ecosystem is changing rapidly.

The strongest direction for the final document is therefore not simply to explain how a spot weld is made. It should explain how a modern robotic welding process is engineered, programmed, validated, monitored, maintained and improved. The standards framework provides the safety and quality foundation; peer-reviewed research provides evidence for process and digitalization developments; and current industry work points toward more connected, data-driven and flexible welding cells.

A disciplined 33-hour workflow provides enough time to research, analyze, draft, validate and professionally format the final document. The proposed roadmap can subsequently be expanded into a plant-neutral technical manual, training module, troubleshooting guide, or digital manufacturing improvement project.

# 15. References

[1] American Welding Society (AWS), D8 Committee on Automotive Welding — published automotive welding standards and committee scope. https://www.aws.org/About/Get-Involved/Committees/D8-Committee-on-Automotive-Welding/

[2] Wang, P. E., et al. (2025). Applicable and generalizable machine learning for intelligent welding in automotive manufacturing. Welding in the World, 69, 1349–1384. https://link.springer.com/article/10.1007/s40194-025-01951-5

[3] Pellegrinelli, S., Pedrocchi, N., Molinari Tosatti, L., Fischer, A., & Tolio, T. (2017). Multi-robot spot-welding cells for car-body assembly: Design and motion planning. Robotics and Computer-Integrated Manufacturing, 44, 97–116. DOI: 10.1016/j.rcim.2016.08.006. https://www.sciencedirect.com/science/article/pii/S0736584515300302

[4] Digitalization of Process Planning of Spot Welding in Body-in-white (2016). Procedia CIRP, 50, 618–623. DOI: 10.1016/j.procir.2016.05.082. https://www.sciencedirect.com/science/article/pii/S2212827116305728

[5] ISO (2025). ISO 10218-1:2025 — Robotics — Safety requirements — Part 1: Industrial robots. https://www.iso.org/standard/73933.html

[6] ISO (2024). ISO 14373:2024 — Resistance welding — Procedure for spot welding of uncoated and coated low-carbon steels. https://www.iso.org/standard/83934.html

[7] ISO (2005). ISO 17657-1:2005 — Resistance welding — Welding current measurement for resistance welding — Part 1: Guidelines for measurement. https://www.iso.org/standard/39996.html

[8] AWS (2021). AWS D8.1M:2021 — Specification for Automotive Weld Quality — Resistance Spot Welding of Steel. https://pubs.aws.org/p/2067/d81m2021-specification-for-automotive-weld-quality-resistance-spot-welding-of-steel

[9] AWS (2026). AWS D8.2M:2026 — Specification for Automotive Weld Quality — Resistance Spot Welding of Aluminum. https://pubs.aws.org/p/2327/d82m2026-specification-for-automotive-weld-quality-resistance-spot-welding-of-aluminum

[10] AWS (2022). AWS D8.9M:2022 — Test Methods for Evaluating the Resistance Spot Welding Behavior of Automotive Sheet Steel Materials. https://pubs.aws.org/p/2095

[11] Failure behaviour of Spot-welds on automotive steel sheets (2022). Materials Today: Proceedings, 62, 6120–6124. DOI: 10.1016/j.matpr.2022.05.020. https://www.sciencedirect.com/science/article/pii/S2214785322032916

[12] Quality Assurance in Resistance Spot Welding: State of Practice, State of the Art, and Prospects (2024). Metals, 14(2), 185. https://www.mdpi.com/2075-4701/14/2/185

[13] Machine learning for predicting resistance spot weld quality in automotive manufacturing (2024). Results in Engineering. DOI: 10.1016/j.rineng.2024.103570. https://www.sciencedirect.com/science/article/pii/S2590123024018139

[14] Luo, R., Zhou, S., Nie, L., et al. (2025). Quality prediction method for automotive body resistance spot welding based on digital twin technology. Scientific Reports, 15, 24391. DOI: 10.1038/s41598-025-09959-z. https://www.nature.com/articles/s41598-025-09959-z

[15] Resistance Welding Quality Through Artificial Intelligence Techniques (2025). Sensors, 25(6), 1744. https://www.mdpi.com/1424-8220/25/6/1744

[16] Influences of Various Parameters on the Weld Spot Diameter during Resistance Spot Welding of the Aluminum Alloy EN AW-6014 (2026). Procedia Structural Integrity, 77, 229–236. DOI: 10.1016/j.prostr.2026.01.031. https://www.sciencedirect.com/science/article/pii/S2452321626000314

[17] American Welding Society Welding Journal (2025). AI in Automotive Welding — virtual sensing, resistance spot welding models and machine learning. https://www.aws.org/magazines-and-media/welding-journal/2025/january/wj-jan-25-feature-01-ghassemi-armaki-gm

[18] An Approach for Expulsion Predicting in Resistance Spot Welding (2024). Procedia CIRP, 130, 1732–1738. DOI: 10.1016/j.procir.2024.10.308. https://www.sciencedirect.com/science/article/pii/S2212827124014665

[19] ISO (2025). ISO 10218-2:2025 — Robotics — Safety requirements — Part 2: Industrial robot applications and robot cells. https://www.iso.org/ics/25.040.30/x/

[20] ISO (1998). ISO 9283:1998 — Manipulating industrial robots — Performance criteria and related test methods. https://www.iso.org/standard/22244.html

# Appendix A — Research Source Matrix

| Source type | Examples used | Best use in final document |
| --- | --- | --- |
| International standards | ISO 10218, ISO 14373, ISO 17657, ISO 9283 | Safety, procedures, measurement, robot performance |
| Professional welding standards | AWS D8.1M, D8.2M, D8.9M | Automotive weld quality and test methods |
| Peer-reviewed research | Robotics/BIW, RSW quality, AI/ML, digital twins | Technical evidence and emerging methods |
| Industry publication | AWS Welding Journal | Current technology and practical trend context |
| Supplier case studies | Digital-twin/offline programming examples | Illustrative implementation only |

# Appendix B — Final Submission Quality Checklist

☐ File is saved as .docx and opens correctly in Microsoft Word.

☐ Title, author and research topic are clearly identified.

☐ Research objective and scope are explicit.

☐ Literature review includes findings, not only a source list.

☐ Current trends are supported by recent sources.

☐ Technical specifications are clearly separated from universal claims.

☐ Standards are identified by exact title and edition/year.

☐ Withdrawn/superseded standards are not presented as current requirements.

☐ In-text citations correspond to the reference list.

☐ Tables are readable and headings are consistent.

☐ Safety statements do not replace plant risk assessment or official procedures.

☐ AI/ML results are described as research findings rather than guaranteed production performance.

☐ Roadmap includes introduction, main sections, conclusion and writing timeline.

☐ Total planned effort is between 30 and 35 hours.

☐ Final proofread completed for spelling, grammar and formatting.

END OF REPORT